THE STATE UNIVERSITY OF NEW YORK AT FREDONIA

PRE-HOT WORK CHECK LIST

Complete this check list and bring it, along with the requested permit, to the office of Environmental Health & Safety, 140 Hendrix Hall at least 24 hours before work is to begin.
This checklist must also be completed each day prior to beginning work. All completed checklists should be turned in to the EH&S office at the end of the project.

Follow these precautions:

- Sprinklers Operational

- Available sprinklers, hose streams and extinguishers are in service and good repair

- Have extinguishers available at work site

- Have work equipment in good repair

- Are covers off smoke detectors

Within 35 foot (11M) of work:

- Wet down combustible floors. Cover with damp sand or other shield.

- Remove flammable liquids, dust, lint, and oily deposits

- Cover all wall and floor openings

- Suspend fire resistant covers beneath work

- Cover or move combustible materials with fire resistive barriers

- Contractor ID issued and current

- Cover smoke heads with covers

- Fire extinguisher available and in place

Work on or near walls or ceilings:

- Move combustibles away from sides of walls

- When working with suspended ceilings be sure to protect concealed spaces

Floors:

- Sweep floor clean of combustibles

Work on enclosed equipment:

- Clean equipment removing all combustibles

- Purge equipment of flammable vapors

Fire watch and work monitoring:

- Fire watch should be provided during the hot work and one half hour after work is completed

- Train fire watch in use of hot work equipment and alarm systems

- A fire watch may be required above, below and in adjacent areas where work takes place

Signature of Supervisor (or designee) ______________________________   Date __________________

Assigned Permit #

HWP-7